U4-AT2 PYTHON CODE

- PYTHON CODE:

# Task 1: Basic Traffic Signal Reinforcement Learning

import random

# Initial traffic conditions

traffic = {

"North": 10,

"South": 8,

"East": 12,

"West": 9

}

print("Initial Traffic:")

for direction, vehicles in traffic.items():

print(direction, ":", vehicles)

# Possible traffic signal actions

actions = ["NS_GREEN", "EW_GREEN"]

# Select an action randomly

action = random.choice(actions)

print("\nSelected Traffic Signal:", action)

# Calculate reward

if action == "NS_GREEN":

passed = traffic["North"] + traffic["South"]

else:

passed = traffic["East"] + traffic["West"]

reward = passed

print("Vehicles Passed:", passed)

print("Reward:", reward)

if reward > 15:

print("Successful traffic signal decision.")

else:

print("Decision needs improvement.")

- PYTHON CODE:

# Task 2: Q-Learning Based Traffic Signal Control

import random

# States represent traffic conditions

states = [

"LOW",

"MEDIUM",

"HIGH"

]

# Actions represent traffic signal choices

actions = [

"NS_GREEN",

"EW_GREEN"

]

# Initialize Q-table

q_table = {}

for state in states:

q_table[state] = {}

for action in actions:

q_table[state][action] = 0.0

# Q-learning parameters

alpha = 0.1

gamma = 0.9

epsilon = 0.2

def get_reward(state, action):

if state == "LOW":

return 10

if state == "MEDIUM":

if action == "NS_GREEN":

return 15

else:

return 8

if state == "HIGH":

if action == "NS_GREEN":

return 20

else:

return 5

# Training

episodes = 1000

for episode in range(episodes):

state = random.choice(states)

# Exploration / exploitation

if random.random() < epsilon:

action = random.choice(actions)

else:

action = max(

actions,

key=lambda a: q_table[state][a]

)

reward = get_reward(state, action)

# Choose next state

next_state = random.choice(states)

max_next_q = max(

q_table[next_state].values()

)

# Q-learning update

old_q = q_table[state][action]

new_q = old_q + alpha * (

reward + gamma * max_next_q - old_q

)

q_table[state][action] = new_q

# Display learned Q-table

print("Learned Q-Table\n")

for state in states:

print(state)

for action in actions:

print(

" ",

action,

"=>",

round(q_table[state][action], 2)

)

best_action = max(

actions,

key=lambda a: q_table[state][a]

)

print(" Best Action:", best_action)

print()

- PYTHON CODE:

# Task 3: Performance Comparison

# Conventional Traffic Signal vs RL Traffic Signal

import random

# Number of vehicles

vehicles = 100

# Generate waiting times for conventional system

conventional_waiting = []

for i in range(vehicles):

waiting_time = random.randint(20, 50)

conventional_waiting.append(waiting_time)

# Generate waiting times for RL system

# RL system is assumed to reduce waiting time

rl_waiting = []

for i in range(vehicles):

waiting_time = random.randint(12, 35)

rl_waiting.append(waiting_time)

# Calculate average waiting time

conventional_average = (

sum(conventional_waiting) / vehicles

)

rl_average = (

sum(rl_waiting) / vehicles

)

# Calculate improvement

improvement = (

(conventional_average - rl_average)

/ conventional_average

) * 100

print("Traffic Signal Performance")

print("--------------------------")

print(

"Conventional System Average Waiting Time:",

round(conventional_average, 2),

"seconds"

)

print(

"RL System Average Waiting Time:",

round(rl_average, 2),

"seconds"

)

print(

"Waiting Time Reduction:",

round(improvement, 2),

"%"

)

if rl_average < conventional_average:

print("\nRL system performs better.")

else:

print("\nConventional system performs better.")

- PYTHON CODE:

# Task 4: Extended Reinforcement Learning Experiment

import random

states = [

"LOW",

"MEDIUM",

"HIGH"

]

actions = [

"NS_GREEN",

"EW_GREEN"

]

# Initialize Q-tables

q_table = {

state: {

action: 0.0

for action in actions

}

for state in states

}

# Parameters

alpha = 0.1

gamma = 0.9

epsilon = 0.3

episodes = 500

rewards_per_episode = []

def calculate_reward(state, action):

if state == "LOW":

return random.randint(8, 12)

elif state == "MEDIUM":

if action == "NS_GREEN":

return random.randint(12, 18)

else:

return random.randint(6, 10)

else:

if action == "NS_GREEN":

return random.randint(18, 25)

else:

return random.randint(4, 8)

# Training process

for episode in range(episodes):

state = random.choice(states)

total_reward = 0

for step in range(10):

# Exploration

if random.random() < epsilon:

action = random.choice(actions)

# Exploitation

else:

action = max(

actions,

key=lambda a: q_table[state][a]

)

# Receive reward

reward = calculate_reward(

state,

action

)

total_reward += reward

# Select next state

next_state = random.choice(states)

# Maximum future Q-value

max_future_q = max(

q_table[next_state].values()

)

# Update Q-value

current_q = q_table[state][action]

q_table[state][action] = (

current_q

+ alpha * (

reward

+ gamma * max_future_q

- current_q

)

)

state = next_state

rewards_per_episode.append(total_reward)

# Calculate average reward

average_reward = (

sum(rewards_per_episode)

/ len(rewards_per_episode)

)

print("Extended RL Experiment")

print("----------------------")

print("Number of Episodes:", episodes)

print(

"Average Reward:",

round(average_reward, 2)

)

print("\nFinal Q-Table:")

for state in states:

print("\nState:", state)

for action in actions:

print(

action,

"=>",

round(q_table[state][action], 2)

)

best_action = max(

actions,

key=lambda a: q_table[state][a]

)

print(

"Recommended Action:",

best_action

)